2026 Logic System Lab

邏輯系統實驗

Lab3 Report

|  | 姓名 | 學號 |
| --- | --- | --- |
| 組員1 | 何家睿 | E24146644 |
| 組員2 | 張家榕 | F84131251 |

組別 :  01

[ Question 1 ]

Prove that Cout derived from the two equations is the same

- Hint : X+XZ=X+Z

- AB=AB+AB

- A∙B+(AB)∙Cin  =>  A∙B+(A+B)∙Cin

A∙B+A⊕B∙Cin

=AB+A'BC+AB'C

=BA+A'C+AB+B'C

=BA+C+AB+C

=AB+BC+AC=AB+(A+B)∙Cin

- A∙B+(A+B)∙Cin  =>  A∙B+(AB)∙Cin

A∙B+(A+B)∙Cin

=AB+BC+AC

=BA+C+AB+C

=BA+A'C+AB+B'C

=AB+A'BC+AB'C=AB+(A'B+AB')=A∙B+A⊕B∙Cin

[ Question 2 ]

Explain the underlying logic behind this equivalence

A∙B+(A+B)∙Cin≡A∙B+(AB)∙Cin

Answer:   By observing the truth tables of XOR and OR, we can see that the only difference between these two logic gates is when A=1and B=1. In this entire function, there is another term AB. Because AB outputs 1 exactly in the case where A=1 and B=1, it already covers this specific condition. Thus, (AB)can be replaced by (A + B).

[ Practice 1 ~ 4 ]

Enter Pass/Fail only (No screenshots required)

|  | pass or fail |
| --- | --- |
| RCA_4 | pass |
| RCA_8 | pass |
| CLL_4 | pass |
| CLA_4 | pass |
| CLA_64 | pass |
| Adder_8_OV | Pass |
| ALU_8 | Pass |

[ Question 3-1 ]

Screenshot the waveform for each case and explain why the values appear as they do

- Case 1 :   A = 4’b1010, B = 4’b0101, Cin = 1’b1

Explanation:

Note that 1010 + 0101 + 0001 = 10000, which triggers an overflow in a 4-bit display (Sum=0000) . However, this specific value was captured by the PG (Propagator-Generator) unit, indicating a carry-out generated during the propagation process.

- Case 2 :   A = 4’b0011, B = 4’b1110, Cin = 1’b0

Explanation:

Note that 0011 + 1110 + 0000 = 10001, which also results in a 4-bit overflow. Consequently, the Sum = 0001. Nevertheless, this operation was recorded by the GG (Group Generate) signal, indicating a carry was generated.

[ Question 3-2 ]

Screenshot the waveform for each case and explain why the values appear as they do

- Case 1 :  A = 64’hFFFF_FFFF_FFFF_FFFF, B = 64’d0, Cin = 1’b1

Explanation:

Similar to Q3-1, adding FFFF,FFFF,FFFF,FFFF and 0000,0000,0000,0001 results in a 65-bit value (1,0000,0000,0000,0000), which causes a 64-bit overflow. Consequently, the Sum remains 0. However, this overflow bit is stored at Cout, which can then be propagated to subsequent modules.

[ Question 4 : Overflow for Adder_8_OV ]

- Please describe when overflow occurs

Answer:  An overflow occurs when both operands (A and B) have the same MSB (sign bit), but the resulting Sum has a different sign bit.

- Please derive the Boolean expression of overflow

- OV= A7+B7+S'7 +A'7+B'7+S7

- assign OV = ((AsignBit & BsignBit & ~SumsignBit) | (~AsignBit & ~BsignBit & SumsignBit));

- assign OV = (AsignBit == BsignBit) & (SumsignBit != AsignBit);

[ Question 5 : Overflow for ALU_8 ]

- Please discuss whether overflow conditions need to be modified

Answer:

Overflow detection is essential for signed addition to ensure the result is correct. However, for unsigned addition, the overflow bit is simply the Carry-out, so it is often considered less of a 'problem' and more of just an extra bit.

- Please derive the Boolean expression of overflow
Answer:

I use Adder_8_OV module , which Logic is:

OV= A7+B7+S'7 +A'7+B'7+S7

[ Question 6 ]

Please explain CARRY4 in FPGA

- How it works ?

CARRY4 是一個 4-bit 的硬體加速單元。它不像 RCA 要在那邊等 Gates 一個接一個傳遞訊號，而是利用 FPGA 內部預先連好的「專用進位線」（Dedicated Carry Chain）。它會同時接收 4 位元的資料，並配合旁邊的 LUT（查找表）產生的 P、G 訊號，瞬間把進位算出來並丟給下一級。

- Its Pros & Cons (優缺點) compared to RCA and CLA

Pros :

- 速度快： 在 FPGA 裡，一般的佈線延遲很大；CARRY4 是快速通道」，專門給進位訊號跑，速度比我們自己用 Gates 寫出來的 RCA 快得多。

- 省算力： 如果自己寫 CLA，會用到超多 LUT 資源，電路會變得很亂。用 CARRY4 的話，它是 FPGA 裡本來就有的閒置專用硬體，是人家寫好的LUT。

Cons :

- 硬體通用性差： 這是我覺得最明顯的缺點。CLA 的概念是通用的，不管在哪都能跑；但 CARRY4 是 Xilinx 特有的。如果今天換成別家的 FPGA，這套邏輯就得重寫，沒辦法直接搬過去。

- 位元固定： 一次只能處理 4 bits。雖然可串聯，但如果要做一些很奇怪位元的運算，就會需要調整一下，不像自己寫的加法器那麼有彈性。

[ Question 7 ]

Check if the synthesis tool utilized a CARRY4 primitive for RCA & CLA, providing the reason why or why not

RCA_8:

Since my design uses gate-level descriptions for the basic units (Half Adder and Full Adder), Vivado treats the circuit as a collection of individual logic gates rather than a high-level addition operation. View to the schematic, proof that synthesis tool does not automatically replace them with the CARRY4 primitive.

CLA_64:

For the same reason as the RCA_8, my CLA design utilizes gate-level descriptions for its two basic units: the PFA and CLL. Maybe the synthesis tool prefert to remaining this manual gate-level structure, it implements the design using LUTs instead of the CARRY4 primitive.

[ 心得 ]

組員一 何家睿 E24146644

In this lab, I gained hands-on experience with adder circuits, which are the fundamental building blocks of a CPU's ALU. By building each module step-by-step in Verilog, I gained a great sense of accomplishment. The most rewarding part was implementing the ALU itself, as it allowed me to integrate everything I learned in this course into a single, functioning system.

組員二 張家榕 F84131251

在本週因為期中考等因素，所以我還沒開始動工，但依照先前我們兩個都喜歡各自做一份自己的報告來看，這次的Lab3 4我也會在截止日前完成一份的，很感謝助教用心做的教學影片，然後也感謝組員讓我可以比較沒有壓力的完成這次的實驗！